Change Language

- 一，The way of change fingerprint scanner language: please insert the USB wire of fingerprint scanner into the computer, and click “Write In Voice”.

- 二, The way of change fingerprint reader language: please insert the special USB wire of fingerprint reader into the computer, and another port insert the USB of fingerprint reader also. (It is power by USB not 12V), then click “Write In Voice”.

- Remark: standard connection of USB(standard USB port, another side use the PH1.25 4P wire)

Write In Voice:

- 1, Open the “FKUserWavedown” setting tool

- 2, Then select the recorded voice content by pressing the key on the right. (It can use our standard recorded sound files or your own recorded language files)

- 3, Click “Communication Setting”, select the USB communication and click “Setting”

- 4, Select “Auto play the voice of download completely.

- 5, Last, click “Write In Setting”.